| Watts |  | Volts | Total Amps (1) | CFM |  | Maximum Air Temperature Rise oF (oC) | Weight lb (kg) | Catalog Number (2) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Min. | Max. |  |  | Min. | Max. |  |  |  |  |
| - | 1500 | 120 | 13.0 | - | 70 | 68 (38) | 14 (6) | 935U01500B¶ |  |
| 750 | 1500 | 120 | 13.0 | 35 | 70 | 68 (38) | 14 (6) | 935U01500B¶-T |  |
| 750 | 1500 | 120 | 13.0 | 35 | 70 | 68 (38) | 14 (6) | 935U01500B¶-TZ |  |
| - | 1500/1125 | 240/208 | 6.7/5.9 | - | 70 | 68/51 (38/28) | 14 (6) | 935U01500V¶ |  |
| 750/563 | 1500/1125 | 240/208 | 6.7/5.9 | 35 | 70 | 68/51 (38/28) | 14 (6) | 935U01500V¶-T |  |
| 750/563 | 1500/1125 | 240/208 | 6.7/5.9 | 35 | 70 | 68/51 (38/28) | 14 (6) | 935U01500V¶-TZ |  |

(1) Total Amps = Heating element amps and motor amps.

(2) ¶ Add W to end of catalog number for white finish, A for almond, or SS for custom stainless steel front cover with black grills.

T = Two-stage built-in thermostat.

TZ = Two-stage built-in thermostat with timer.

Architect’s and Engineer’s Specifications

Heating Elements – Stitched ribbon-type element.

Outlet Grille – The 20-gauge steel grille has a polyester powder paint finish available in white, almond, or optional stainless steel front cover.

Built-in Controls – Automatic reset thermal cutout prevents overheating.

Fan Motor – Whisper-quiet squirrel-cage fan operates at 33 dBA with 35 CFM and 42.5 dBA with 70 CFM.

Installation Requirements – The air outlet should be installed a minimum of 12-inches above vinyl floors.

Rough In Dimensions – All heaters: 11-5/8” wide x 15-1/2” high x 2” deep.